// board:  LilyGo T-Display

// port : com3

//

//Library:

// 1. DHT sensor library by adafruit

// 2. TinyGSM by Volodymyr Shymanskyy

// 3. WiFiManager by tzapu

//

// ESG Sensor Data Capture and send to Thinkspeaks Via GSM or Wifi

// Coded by IT Team KISMEC

// Coder 01 : Override

// Coder 02 : A-win

// Coder 03 : Kinah

#include <WiFi.h>

#include <HTTPClient.h>

#include <WiFiManager.h> // WiFiManager library

#include "DHT.h"

#define TINY_GSM_MODEM_SIM800 // Define GSM modem model

#include <TinyGsmClient.h>

// Choose connection type

bool useGSM = false;  // Set to true to use GSM

bool useWiFi = true; // Set to true to use WiFi

// ---------- Start Initilise ----------------------//

// GSM settings

#define GSM_BAUD 9600

const char* apn = "internet"; // Replace with your GSM provider's APN

const char* gprsUser = "";    // Leave blank if not required

const char* gprsPass = "";    // Leave blank if not required

// GSM setup

HardwareSerial SerialAT(1); // Use UART1 for GSM communication

TinyGsm modem(SerialAT);

TinyGsmClient gsmClient();

// ThingSpeak API details

const char* api_key = "MOC3KU8DJ7NXRGWR";    // Your ThingSpeak API Key

const char* server = "http://api.thingspeak.com";

// DHT sensor setup

#define DHTPIN 4          // GPIO pin where DHT22 is connected

#define DHTTYPE DHT22     // Specify sensor type (DHT22)

DHT dht(DHTPIN, DHTTYPE);

// MQ135 setup

#define MQ135_PIN 34      // GPIO pin where MQ135 is connected (Analog pin)

float airQuality = 0;   // Air quality percentage

// ---------- End Initilise ----------------------//

// ---------- Start Setup Communication----------------------//

// Setup WiFi connection using WiFiManager

void setupWiFi() {

WiFiManager wifiManager;

// Uncomment the line below to reset settings (useful for testing)

// wifiManager.resetSettings();

// AutoConnect starts a portal if Wi-Fi credentials are missing

if (!wifiManager.autoConnect("AutoConnectAP")) {

Serial.println("Failed to connect to Wi-Fi. Restarting...");

ESP.restart();

}

Serial.println("Wi-Fi connected.");

Serial.print("IP Address: ");

Serial.println(WiFi.localIP());

}

// Setup GSM connection

void setupGSM() {

SerialAT.begin(GSM_BAUD, SERIAL_8N1, 16, 17); // RX = GPIO16, TX = GPIO17

Serial.println("Initializing GSM...");

if (!modem.restart()) {

Serial.println("Failed to restart modem!");

while (true);

}

Serial.print("Connecting to GSM network...");

if (!modem.gprsConnect(apn, gprsUser, gprsPass)) {

Serial.println("Failed to connect to GSM!");

while (true);

}

Serial.println("GSM connected.");

}

// ---------- End Setup Communication----------------------//

// ---------- Start Function ----------------------//

// Function to read air quality from MQ135 sensor

float readAirQuality() {

int analogValue = analogRead(MQ135_PIN); // Read the analog value from MQ135

// Convert the analog value to a voltage

float voltage = analogValue * (3.3 / 4095.0); // Assuming 3.3V ADC reference

// Map the analog value to an air quality percentage (this is just a rough mapping)

float airQuality = map(analogValue, 0, 4095, 0, 100); // 0 to 100% air quality

return airQuality;

}

// Function Send data using WiFi

void sendDataWiFi(float temperature, float humidity, float airQuality) {

HTTPClient http;

String url = String(server) + "/update?api_key=" + api_key +

"&field1=" + String(temperature) +

"&field2=" + String(humidity) +

"&field3=" + String(airQuality);

http.begin(url);

int httpResponseCode = http.GET();

if (httpResponseCode > 0) {

Serial.print("Data sent to ThingSpeak via WiFi. HTTP Response code: ");

Serial.println(httpResponseCode);

} else {

Serial.print("Error sending data via WiFi: ");

Serial.println(httpResponseCode);

}

http.end();

}

// Function Send data using GSM

void sendDataGSM(float temperature, float humidity, float airQuality) {

String url = String(server) + "/update?api_key=" + api_key +

"&field1=" + String(temperature) +

"&field2=" + String(humidity) +

"&field3=" + String(airQuality);

// Start by establishing a connection using AT commands

if (modem.isNetworkConnected()) {

Serial.println("GSM network connected");

// Open a TCP connection to the server

modem.sendAT("+CIPSTART=\"TCP\"," + String(server) + ",80");

delay(2000);  // Wait for connection to be established

// Send the HTTP GET request to ThingSpeak

modem.sendAT("GET " + url + " HTTP/1.1\r\n");

modem.sendAT("Host: " + String(server) + "\r\n");

modem.sendAT("Connection: close\r\n\r\n");

// Wait for the server to respond

delay(5000); // Adjust as needed based on your GSM network speed

// Close the connection

modem.sendAT("+CIPCLOSE");

Serial.println("Data sent to ThingSpeak via GSM.");

} else {

Serial.println("GSM network not connected.");

}

}

// ---------- End Function ----------------------//

void setup() {

Serial.begin(115200);

dht.begin();

// Initialize MQ135 sensor

pinMode(MQ135_PIN, INPUT); // Set MQ135 pin as input

if (useWiFi) {

setupWiFi();

} else if (useGSM) {

setupGSM();

} else {

Serial.println("No connection method selected!");

while (true);

}

}

void loop() {

// Read temperature and humidity

float temperature = dht.readTemperature();

float humidity = dht.readHumidity();

// Check if the readings are valid

if (isnan(temperature) || isnan(humidity)) {

Serial.println("Failed to read from DHT sensor!");

delay(2000);

return;

}

// Read air quality from MQ135

airQuality = readAirQuality();

// Check if air quality reading is valid

if (isnan(airQuality)) {

Serial.println("Failed to read from air quality sensor!");

delay(2000);

return;

}

// Log sensor data

if (useWiFi) {

Serial.print("Connection: Wifi :");

Serial.print(WiFi.localIP());

} else if (useGSM) {

Serial.print("Connection: Wifi :");

} else {

Serial.print("No connection method selected!");

}

Serial.print(" Temperature: ");

Serial.print(temperature);

Serial.print(" °C, Humidity: ");

Serial.print(humidity);

Serial.print(" %, Air Quality: ");

Serial.print(airQuality);

Serial.println(" %");

// Send data to thinkspeak

if (useWiFi && WiFi.status() == WL_CONNECTED) {

sendDataWiFi(temperature, humidity, airQuality);

} else if (useGSM) {

sendDataGSM(temperature, humidity, airQuality);

}

// Wait 30 seconds before the next update

delay(30000);

}