// ESG Sensor Data Capture and send to ThingSpeak

// Via GSM or WiFi

// Coded by IT Team KISMEC

// Coder 01 : Override

// board:  LilyGo T-Display

// port : com3

//

//Library:

// 1. DHT sensor library by adafruit

// 2. TinyGSM by Volodymyr Shymanskyy

// 3. WiFiManager by tzapu

//

// Serial Output

// Open Serial Monitor

// Set baud rate to 115200 or 9600

//

// pin 4 = DHT 22

// pin 34 = Air quality Sensor

#include <WiFi.h>

#include <HTTPClient.h>

#include <WiFiManager.h>

#include "DHT.h"

#define TINY_GSM_MODEM_SIM800

#include <TinyGsmClient.h>

// --- Select connection type ---

bool useGSM = false;   // Set true to use GSM

bool useWiFi = true;   // Set true to use WiFi

// --- GSM Configuration ---

#define GSM_BAUD 9600

const char* apn = "internet";  // Your GSM provider's APN

const char* gprsUser = "";     // Leave empty if not needed

const char* gprsPass = "";

HardwareSerial SerialAT(1); // UART1 for GSM (RX=16, TX=17)

TinyGsm modem(SerialAT);

TinyGsmClient gsmClient(modem);

// --- ThingSpeak API ---

const char* api_key = "MOC3KU8DJ7NXRGWR";

const char* server = "api.thingspeak.com";

// --- DHT Sensor ---

#define DHTPIN 4

#define DHTTYPE DHT22

DHT dht(DHTPIN, DHTTYPE);

// --- MQ135 Sensor ---

#define MQ135_PIN 34

float airQuality = 0;

// --- WiFi Setup ---

void setupWiFi() {

WiFiManager wifiManager;

if (!wifiManager.autoConnect("AutoConnectAP")) {

Serial.println("Failed to connect to WiFi. Restarting...");

ESP.restart();

}

Serial.println("WiFi connected.");

Serial.print("IP: ");

Serial.println(WiFi.localIP());

}

// --- GSM Setup ---

void setupGSM() {

SerialAT.begin(GSM_BAUD, SERIAL_8N1, 16, 17);

Serial.println("Initializing GSM...");

if (!modem.restart()) {

Serial.println("GSM restart failed!");

while (true);

}

Serial.print("Connecting to GSM network...");

if (!modem.gprsConnect(apn, gprsUser, gprsPass)) {

Serial.println("GSM GPRS connect failed!");

while (true);

}

Serial.println("GSM connected.");

}

// --- Read Air Quality ---

float readAirQuality() {

int analogValue = analogRead(MQ135_PIN);

float airQuality = map(analogValue, 0, 4095, 0, 100); // 0 to 100%

return airQuality;

}

// --- Send data via WiFi ---

void sendDataWiFi(float temperature, float humidity, float airQuality) {

HTTPClient http;

String url = String("http://") + server + "/update?api_key=" + api_key +

"&field1=" + String(temperature) +

"&field2=" + String(humidity) +

"&field3=" + String(airQuality);

http.begin(url);

int httpResponseCode = http.GET();

if (httpResponseCode > 0) {

Serial.print("WiFi → ThingSpeak HTTP Response code: ");

Serial.println(httpResponseCode);

} else {

Serial.print("WiFi → Error sending data: ");

Serial.println(httpResponseCode);

}

http.end();

}

// --- Send data via GSM ---

void sendDataGSM(float temperature, float humidity, float airQuality) {

if (!modem.isNetworkConnected()) {

Serial.println("GSM not connected.");

return;

}

TinyGsmClient client(modem);

String url = "/update?api_key=" + String(api_key) +

"&field1=" + String(temperature) +

"&field2=" + String(humidity) +

"&field3=" + String(airQuality);

if (!client.connect(server, 80)) {

Serial.println("GSM → Connection to ThingSpeak failed!");

return;

}

client.print(String("GET ") + url + " HTTP/1.1\r\n" +

"Host: " + server + "\r\n" +

"Connection: close\r\n\r\n");

Serial.println("GSM → Data sent to ThingSpeak");

// Optional: read and print response

unsigned long timeout = millis();

while (client.connected() && millis() - timeout < 5000) {

while (client.available()) {

char c = client.read();

Serial.write(c);

}

}

client.stop();

}

// --- Setup ---

void setup() {

Serial.begin(115200);

dht.begin();

pinMode(MQ135_PIN, INPUT);

if (useWiFi) {

setupWiFi();

} else if (useGSM) {

setupGSM();

} else {

Serial.println("No connection method selected!");

while (true);

}

}

// --- Main Loop ---

void loop() {

float temperature = dht.readTemperature();

float humidity = dht.readHumidity();

if (isnan(temperature) || isnan(humidity)) {

Serial.println("DHT sensor read failed!");

delay(2000);

return;

}

airQuality = readAirQuality();

if (isnan(airQuality)) {

Serial.println("MQ135 read failed!");

delay(2000);

return;

}

// Log readings

Serial.print("Temp: ");

Serial.print(temperature);

Serial.print(" °C, Humidity: ");

Serial.print(humidity);

Serial.print(" %, Air Quality: ");

Serial.print(airQuality);

Serial.println(" %");

if (useWiFi && WiFi.status() == WL_CONNECTED) {

sendDataWiFi(temperature, humidity, airQuality);

} else if (useGSM) {

sendDataGSM(temperature, humidity, airQuality);

} else {

Serial.println("No active connection method.");

}

delay(30000); // Wait 30 seconds before next reading

}